| Purpose: | To establish a task-level method for breaking work into steps, identifying hazards, determining controls and communicating a safe work sequence before execution. |
| --- | --- |
| Scope: | This procedure applies to non-routine, complex, high-risk, unfamiliar or changed work and other activities designated by HIRARC, method statement, PTW, client or legal requirements. |
| Duties & Responsibilities | (1) Project Manager (PM): Ensures JSA requirements are integrated into project planning. (2) Safety and Health Officer (SHO): Maintains methodology and facilitates high-risk JSA. (3) Site Safety Supervisor (SSS): Coordinates JSA preparation, briefing and field verification. (4) Project Supervisor (PS): Leads task breakdown and implements controls. (5) Workers / Specialists: Contribute task knowledge and confirm practical controls. (6) Subcontractor Supervisor: Prepares and implements JSA for subcontracted work subject to Company review. |
| ISO Reference: | MS ISO 45001:2018 Clause 6.1.2 |

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Revision No. | Revision Date | Originator | Description of Changes |
| 0 | 01 Sept 2026 | ISO Manager | Initial issue for implementation of MS ISO 45001:2018 Clause 6.1.2 |

| Abbreviation & Definition |  |
| --- | --- |
| HIRARC | Hazard Identification, Risk Assessment and Risk Control |
| JSA | Job Safety Analysis |
| MOC | Management of Change |
| PIC | Person in Charge |
| PM | Project Manager |
| PPE | Personal Protective Equipment |
| PTW | Permit to Work |
| SHO | Safety and Health Officer |
| SSS | Site Safety Supervisor |
| SWI | Safe Work Instruction |

| Step | Activity | PIC | Reference |
| --- | --- | --- | --- |
| 1 | Determine Need and Form the Team 1.1 Triggers Prepare JSA for high-risk, non-routine, complex, first-time or changed tasks; tasks with incident history; or work subject to PTW. 1.2 Team Include the supervisor/work leader, persons performing the task, relevant subcontractor and HSE/technical specialists. 1.3 Information Review HIRARC, drawings, method statement, manuals, SDS, permits, incidents, site conditions and interface information. | PM / SHO / SSS / PS | OCS-S-PRO-02; Method Statement |
| 2 | Define the Task and Work Sequence 2.1 Scope State the exact task, location, boundaries, equipment, crew, prerequisites and exclusions. 2.2 Steps Break work into logical observable steps in the order performed without making steps too broad or excessively detailed. 2.3 Interfaces Identify preceding/following work, simultaneous operations, energy sources, access and handover points. 2.4 Walk-through Where practicable, inspect the actual workplace with the team before approval. | PS / SSS | Project Programme; Site Plan |
| 3 | Identify Hazards for Each Step 3.1 Hazard Prompt For each step consider contact with energy, falls, struck-by/caught-between, lifting, electricity, pressure, fire, substances, ergonomics, environment and human factors. 3.2 Conditions Consider normal, abnormal and emergency conditions, changes in weather, lighting, congestion, isolation and nearby activities. 3.3 Affected Persons Consider the work crew, other contractors, visitors and public. 3.4 Worker Input Record concerns and practical knowledge from persons who perform the task. | SHO / SSS | OCS-S-PRO-02; Incident Lessons |
| 4 | Determine Controls and Safe Sequence 4.1 Hierarchy Apply elimination, substitution, engineering, administrative controls and PPE in order of effectiveness. 4.2 Critical Controls Clearly identify controls whose failure could cause a fatal or serious event and define verification. 4.3 Responsibilities Assign actions and in-task checks to named roles. 4.4 Emergency / Rescue Include emergency stop, communication, rescue and recovery arrangements where relevant. | SHO / PS | PD-S-PRO-03; PD-S-PRO-04; PD-S-PRO-10 |
| 5 | Review, Approval and Permit Linkage 5.1 Review Verify alignment with HIRARC, method statement, SWI, drawings, manufacturer requirements and legal/client controls. 5.2 Approval The supervisor and SSS/SHO shall approve; high-risk work also requires the designated PM/authorising person. 5.3 PTW Where PTW applies, the approved JSA shall support but not replace permit checks, isolations and authorisation. 5.4 Outstanding Actions Work shall not start until prerequisite actions and critical controls are complete. | PS / SSS / SHO / PM | PD-S-PRO-08; PD-S-PRO-07 |
| 6 | Briefing and Field Implementation 6.1 Pre-task Briefing Brief every affected worker in a language and form understood; confirm roles, sequence, hazards, controls and stop-work criteria. 6.2 Acknowledgement Record attendance and understanding; new or replacement crew members shall be briefed before joining. 6.3 Field Verification Confirm actual conditions and controls immediately before starting and throughout the task. 6.4 Deviation Stop work and review the JSA when conditions or methods differ from the approved analysis. | PS / SSS | Toolbox Record; PTW |
| 7 | Review, Closure and Records 7.1 Review Triggers Review after change, incident, near miss, worker concern, control failure, interruption or new information. 7.2 Task Close-out Confirm energy state, housekeeping, barriers, tools, waste and handover before closure. 7.3 Learning Capture improvements for HIRARC, SWI, method statement and future work. 7.4 Retention Retain approved JSA, briefings, revisions, permits and close-out evidence. | PS / SSS / SHO / PM | OHSMS-10V; OHSMS-10W; OHSMS-07O |

| Verified By: Name: Eng Choon Leong Appointment: Executive Director Date: 01 Sept 2026 | Approved By: Name: Amos Haw Chee Seng Appointment: Managing Director Date: 01 Sept 2026 |
| --- | --- |

End